Configuration of the Scales

# A & D Scales

The scales must be configured to output data in grammes, in stream mode, at 9600 baud, 8 bits no parity. These are set using the scales' menu system and the following parameters.

Function		Value

Prt			0

bPS			2

btPr			2

Unit			g

Press and hold “SAMPLE”	The screen will display Func

Press “PRINT”			The screen will display	PoFF  0

Press “SAMPLE”		Until the required parameter is displayed

Press “RE-ZERO”		To change the parameter

Press “SAMPLE”		To move on to other parameters …

Press “PRINT”			To save the changes. The screen will display End

Press “MODE”			To revert back to the weighing screen

# Kern & Sohn Scales

The scales must be configured to output data in grammes, in stream mode, at 9600 baud, 8 bits no parity. These are set using the scales' menu system and the following parameters.

Function		Value

## Weighing Units		g

Press and hold PRINT		Hold until Unit is displayed

Press SET/M			Shows current setting

### Press MODE/CAL		Until it shows “g”

Press SET/M			To Save

Data Transfer Mode	AU PC

Press and hold PRINT		Hold until Unit is displayed

Press MODE/CAL		Pr

Press SET/M			Shows current setting

Press MODE/CAL		Until it shows AU PC

Press SET/M			To Save

Baud Rate		9600

Press and hold PRINT		Hold until Unit is displayed

Press MODE/CAL		Pr

Press MODE/CAL		LAPr

Press MODE/CAL		bAUd

Press SET/M			Shows current speed

Press MODE/CAL		Until it shows 9600

Press SET/M			To save

Use of StockManager

Stockmanager must be used to set up the empty and full weights for each product. This is done in Audit Setup / Product Weights. Once done in one outlet, the weights can be automatically copied to other outlets, as long as consistency in product codes has been maintained.

Use of StockMate

Using StockMate in conjunction with the weighing scales is very similar to normal use of StockMate - no functionality has been lost.

When a product has been selected, either by typing in the product code or by scanning a barcode, the Items prompt is displayed. At this stage, a 'weigh' can be initiated by pressing the W key.

If the product is one that can be weighed, i.e. it is a single product (not a dozens) and it has empty/full weights defined, the actual weight will be transformed into a decimal on the screen. You should then press Enter to accept this.

If you try to weigh a ‘dozens’ product a message will be displayed.

If you weigh something that is lighter than the empty weight or heavier than the full weight, a message will be displayed and the decimal will not be calculated.

If you do not have the serial cable to connect StockMate to the scales, you can type in the weight. At the Items prompt, type in the weight in grammes followed by a 'g', e.g. "579g", and press Enter. This is the equivalent of pressing the W key for an automatic weight.